Smart Socks for Physical Activity Recognition

ELEC-E7840 Smart Wearables — Topic 3

PRELIMINARY — Please review and bring feedback to our next meeting. All assignments are open for discussion.

# Requirements

As a 3-person team, we must recognize all five activity categories:

| Walking | Forward/backward + step count |
| --- | --- |
| Stairs | Up/down + step count |
| Sitting | Feet on floor vs. cross-legged |
| Sit-to-stand | Detection + timing |
| Standing | Upright vs. leaning L/R |

Constraint: Textile sensors only (pressure, bend, strain). No IMUs.

# Team

| Member | Strengths | Focus |
| --- | --- | --- |
| Saara | ML, biosignals, crafts | Features, model, sensors |
| Alex | Prototyping, user testing | Integration, wearability |
| Jing | Systems, EE, hardware | Circuit, ESP32, coordination |

# Technical Approach

Sensing: 5 pressure zones per foot (heel, arch, metatarsal medial/lateral, toe) → 10 channels total

Pipeline: Piezoresistive sensors → ESP32S3 ADC → Computer → ML classification

Output: Activity label + step count in real-time

# Phases

## Phase 1: Sensor Characterization

| Build test sensor (2×2cm piezoresistive sandwich) | Saara |
| --- | --- |
| Set up ESP32 + Arduino environment | Jing |
| Wire voltage divider circuit | Jing |
| Run characterization (weights, response time) | All |
| Analyze and document results | Saara + Alex |

## Phase 2: Prototype Design

| Finalize sensor count and placement | All |
| --- | --- |
| Cut sensor pieces for both socks | Saara |
| Design sock integration + wire routing | Alex |
| Research reference designs from reading list | Alex |
| Build prototype v1 | Saara + Alex |
| Implement full circuit (all channels) | Jing |

## Phase 3: Data Collection (Mid-term)

| Write Arduino + Python data collection software | Jing |
| --- | --- |
| Define collection protocol (activities, reps) | Saara |
| Recruit 8-9 subjects (6 train, 3 test) | All |
| Run collection sessions with video sync | All |
| Organize and validate data | Saara |

## Phase 4: ML & Integration (Period IV)

Note: Saara and Alex complete Period III only. Jing leads these tasks.

| Feature engineering | Jing |
| --- | --- |
| Model training and validation | Jing |
| Real-time classification pipeline | Jing |
| Step counting algorithm | Jing |
| End-to-end system integration | Jing |

## Phase 5: Testing & Final

| Test on held-out subjects | Jing |
| --- | --- |
| Wearability study (WEAR + SUS, 5+ users) | Jing |
| Confusion matrix and analysis | Jing |
| Final presentation | Jing |

# Materials

| Item | Status |
| --- | --- |
| Piezoresistive fabric | Have (~16×16cm) |
| Conductive fabric | Have (2 pieces) |
| Conductive thread | Have |
| ESP32S3 XIAO | Have (1) |
| Breadboard + wires | Have |
| Resistors | Have (verify values) |
| Sock | Have (1, need second) |
| Multimeter | Available at school |

# For Discussion

· Are the role assignments reasonable?

· Meeting schedule: weekly in-person + async?

· Shared workspace: Google Drive, Notion?

· Do we have access to enough test subjects?

· Start with 5 sensors per foot, or simpler?

· Contingency if sensor characteristics are poor?

# Before Next Meeting

All: Review this document, prepare feedback

Jing: Set up ESP32 environment, verify basic serial communication

Saara: Prepare to build test sensor at next session

Alex: Find 2-3 reference smart sock designs from course reading list