Meeting 21.1.2026, all present

- Talking about how to approach the problem: mapping out different sensor types available, user study, how to use the sensors to create the signals we need

- Alex: sock design, asking physiotherapists etc., usability, user requirements

- Saara: biosignal creation, knee pads/socks

- Jing: preliminary analysis on the electronics side

- Idea: not only a sports device but also hydration measurement

- Tasks: fabrication, electronics, data, ...

What characterizes the movements we need to detect:

- Walking on a flat surface

  - Walking forward: pressure on the heel, pressure on the ball of the foot, front of the knee pad stretches

  - Walking backward: pressure on the ball, pressure on the heel, front of the knee pad stretches

- Climbing stairs

  - Up: pressure on the heel -> ball (can be rather simultaneously), front of the stepping foot knee pad stretches at first (socks and knee pads) -> could be separated from walking on a flat surface by the stronger stretching of the knee pad front

  - Down: pressure on the ball -> heel, front of the knee pad of the leg that is behind stretches

- Sitting on a chair

  - Both feet on the floor: equal pressure under both heels, equal stretch of the knee pad fronts

  - Legs crossed: Pressure only under one heel, stretch on both knee pads but the magnitude is different

- Sit-to-stand

  - Pressure under both heels increases and the stretch of the knee pads decreases

- Standing

  - Upright: Equal pressure under both heels and balls, no stretching of the knee pads

  - Leaning to one side: More pressure on one side, no or very little stretching of the knee pads

Required sensors:

2 pressure sensors per sock, one under the heel and the other one under the ball (heat-pressed?) = altogether 4 needed, aim at making them as identical as possible for improved comparability

1 stretch sensor per knee pad, positioned in front of the knee (knitted?)

Open questions:

How do we structure the knee pad to fit different people? Option: knitted stretch part in the middle, connect to stretchy fabric that can be adjusted to the “starting position” by straps to minimize the impact of different leg diameters to the results

How do we position the pressure sensors such that the sock works for as many people as possible considering its size

References:

Approach with 3 pressure sensors per foot for skating technique detection: https://www.frontiersin.org/journals/sports-and-active-living/articles/10.3389/fspor.2025.1554264/full (2 at ball, 1 at heel)

Approach with 5 pressure sensors per foot for healthcare and sport monitoring: https://pubs-acs-org.ezproxy.utu.fi:2443/doi/full/10.1021/acsnano.8b08329 (ball, heel, big toe, sides)